NU–IACUC POLICY

Northeastern University Institutional Animal Care and Use Committee

# Tips Only Rodent Surgery

Approved: 03/11/2025

All survival rodent surgery at Northeastern University will be performed aseptically which adheres to the specifications as outlined in The Guide for the Care and Use of Animals.  This policy allows researchers to use TIPS only procedures for performing rodent surgeries.  Meaning the focus is keeping the “tips” of the surgical instruments sterile with the handles are not.

The Tips process allows the surgeon to not wear sterile gloves and to use a glass bead sterilizer (GBS) for sterilizing instruments and any item that can be sterilized with heat, i.e. a drill bit.  If properly followed, this allows the surgeon to keep the surgical field sterile, thus limiting the possibility of infection.

Surgery Location

Surgery must take place in a work area or laboratory removed from the mainstream of traffic.  The area must be uncluttered and sanitizable.  It is not recommended to perform surgery directly on a table or bench surface without covering the area with a plastic backed absorbent pad for sanitation reasons and warmth for the animal.  A disposable absorbent chuck fulfills this requirement.

Surgical Instruments and Implantable Devices

All surgical instruments must be sterilized prior to use in either a GBS or in steam or gas sterilizer.  The GBS must be turned on 15-30 minutes before surgery.  Multiple surgeries require instruments be sterilized in between each surgery in a GBS.  Instruments should be placed in the GBS for 15 seconds.  To prevent the possibility of burning the animal, ensure the tips have completely cooled before using in an animal.

After sterilization, instruments must by placed on a sterile surgical tray that has a delineation of sterile and not sterile.  This ensures the surgeon does not contaminate the tray and the tips of the instruments stay sterile.  Following surgery, all organic matter should be removed from the instruments and placed in a pack for re-sterilization.

All implantable devices and materials, including catheters, mini-pumps, tubing, screws, etc. must be either sterilized via gas or steam sterilization prior to use.

Required Garments for performing rodent surgery

- A clean yellow animal gown, surgical mask, safety glasses, and clean gloves must be worn.

Please follow these pre-incision steps listed below for “Tips” rodent surgery:

- Weigh the animal.

- Administer analgesics to the animal prior to the start of surgery (i.e. prior to making the skin incision).  For the proper dose of analgesic, see the NU-IACUC policy on the Recommended Doses Anesthetics and Analgesics of Laboratory Animals.

- According to body weight, anesthetize the animal with the appropriate dose of approved anesthetic.  (see the NU-IACUC policy on the Recommended Doses Anesthetics and Analgesics of Laboratory Animals).

- Prepare the animal for surgery

- Apply ophthalmic ointment to anesthetized animal’s eyes to prevent ocular drying and potential irritation or ulceration.

- Remove the hair at the intended incision site using clippers (preferred), or depilatory cream, prior to placement of the animal on the surgery table. The area of hair removal must be at least 150% the size of the incision in all directions.

- Disinfect the skin at the intended incision site by first scrubbing the area with povidone/iodine or appropriate disinfectant followed by a rinse with 70% alcohol and repeat three times.  Do not touch the incision site with your gloves or hands.  Only with surgical instruments.

- During anesthesia and surgery, the animal must be kept warm with a heat source.  Warmth can be maintained using various heat sources.  Circulating water heating pads or specific temperature-controlled heating platforms are preferred over electric pads and heating lamps.  Close attention must be paid as not to overheat the animal.  Placing a blue chuck over the pad will help with this.  Electric heat pads are not recommended, but if used the pad must be covered with an absorbent pad and its temperature must be closely monitored as not to overheat the animal.

- Remember, all anesthetics and many surgical procedures will result in heat loss, and body temperature management is an important factor in survival and recovery.

- Additionally, failure to provide adequate thermal support will reduce the effectiveness of anesthetics and analgesics.

- Draping of the animal is required for “Tips” surgery.  The drape must be sterile but does not have to cover the whole animal but should cover the area around the incision.  This is required especially for when the surgeon is suturing the animal as not to contaminate the sutures.  When handling the drape, only handle the outside corners.

Guidelines and Reminders for Tips Surgery:

- The surgery must be performed aseptically using “tips” techniques.

- The surgeon may only use the tips of the instruments.

- The sterile tips are an extension of your fingers and hands.

- The surgeon may not touch the incision site with their fingers.

- The surgeon may only touch the non-sterile handles of the instruments, and they may not touch the sterile tips.

- We suggest using a disposable surgical blade and handle.  However, if that is not possible, surgeons must attach scalpel blades to the handles using hemostats or needle holders.

- If the tips of the instruments touch something non-sterile, they must be re-sterilized in the GBS.

- Sterile instrument tips may only be used to handle any sterile supplies within your sterile field.

- The GBS must be used in between animals or if the tips are no longer sterile.

- When opening sterile packs and instrument trays, cautioned must be followed as not to contaminate the sterile instruments.

- Drill handles, microscopes and stereotaxic apparatus do not have to be sterile.

- The surgeon may not touch the suture material with their hands, they are to use needle holders and forceps/hemostats. Ensure the suture material stays on the sterile drape while suturing.

- Incision site closure must be done using either sutures, stainless steel wound clips, surgical glue, or a combination of these.  The selection of suture material should be based on the properties of the material and characteristics, i.e. absorbable vs. non-absorbable.  The use of silk or braided sutures in the skin is not allowed due to the possibility of wicking of bacteria into the incision site through the braided material.  A monofilament suture material or sterile wound clips must be used for skin closure.

Multiple Surgeries:

When performing multiple surgeries during a single session, care must be taken to avoid contaminating one animal to another.  Instrument tips should be wiped of organic material or tissue and must be placed in a GBS for 15 seconds in between animals.  When using the GBS, do not leave the instruments in too long or they will be too hot to handle.

Post-Surgical Care/Monitoring

The Principal Investigator, or designate, is responsible for the monitoring of animals recovering from anesthesia and surgery.

Immediate post-surgery period and up to 7 days post surgically:

- The animal cannot be left unattended and must be monitored until it is ambulatory (walking).  Only then can it be returned to the animal housing room.  During the recovery process the animal should be turned from side to side every 10-15 minutes until the animal is sternal.

- To maintain the body temperature and hasten recovery, the animal must be kept warm.  Place a drape, blanket or pad between the animal and the heat source to prevent overheating and/or burns.

- Animals recovering from anesthesia should not be placed directly onto contact bedding, i.e. pine shavings, as they may inhale or ingest bedding particles.  Placing the animal on a paper towel or equivalent should prevent this from happening.

- Do not place anesthetized animals with conscious animals.  House separately until anesthetized animal is completely recovered.

- For all surgeries you must complete a Rodent Surgery Card and place on the cage.  This card is displayed as the front cage card for 7-10 days post-surgery.

- The Rodent Surgery Card has sections for the following information:

- Surgery and date performed,

- Anesthetic used.  There is also a section for the PI to record if/when analgesia is given after the first initial dose.

- Post-Surgical monitoring (on back of card):  For the first 3 days, the animal(s) are observed daily for activity, pain, and infection at the incision site.  There is a scoring system which is listed on the card for each category.  Pain evaluation should be based on The Prevention, Recognition, and Treatment of Pain & Distress in Laboratory Animals.  The incision site should be observed for cleanliness, healing and dehiscence.

- After 7-10 days, the card is placed behind the original cage card.  If the animal is permanently disabled, the card must remain displayed on the cage.  This card should not be used as the animal’s primary cage ID card.

- Evaluate adequate hydration by “tenting” loose skin between the shoulder blades.  Skin should normally fall back immediately to the body when tented.  The absence of this response may indicate the animal is dehydrated and is in need of intravenous or subcutaneous fluid therapy (consult veterinarian for best type, dose, and route of fluids).

- Evaluate adequate nutrition by palpating spine and/or ribs and evaluation the animal for body condition.  A body Condition Score of <2 will require euthanasia.  Weighing the animal as described in the protocol is required.  A greater than 15% loss of body weight would require the animal be euthanized.

- Sutures or wound clips must be removed 7 – 10 days post-op.

Rodent Surgery Card

### Rodent Surgery Card P.I._________________________ PROTOCOL #_______________ Contact Person/Phone #____________________________________ Date & Procedure Performed:________________________________ Anesthetic Iso Ket/Xyl Other___________ Post-op Analgesic Buprenorphine ___mg/kg Route____ Time____ Other________-mg/kg Route____ Time____ Exempt per IACUC Comments: _____________________________________________ _______________________________________________________ _______________________________________________________ # (front)

Surgery Score Code: Activity 0=normal 1=reduced 2=poor 3=moribund Pain 0=none 1=mild 2=moderate 3=severe Incision 0=normal 1=inflamed 2=infected 3=dehisced Post op care; Day 0; Day 1; Day 2; Day 3. Activity. Pain. Incision. Initials. Analgesic; am pm; am pm; am pm; am pm. Comments: __________________________________________ _________________________________________________________ (Back)